LIST OF EXPERIMENTS

1. Zener Diode Characteristics

2.	Rectifiers with out filter

i. Half Wave Rectifier (HWR)

ii. Full Wave Rectifier (FWR)

iii. Bridge Rectifier

3.	BJT Characteristics (Common Emitter Configuration)

4.	JFET Characteristics

5. Operational Amplifier Frequency Response

Characteristics

Zener Diode

Expt.No.1

Aim: To determine Zener  break down voltage, Zener resistance from Zener diode characteristics and to determine Zener voltage regulation characteristics for a given Zener diode (IN4723A)

Apparatus:

Zener diode circuit kit,

Ammeters (0-50mA),

Voltmeter(0-10v),

Rheostat(0-1180 Ω),

Digital Multi Meter(DMM),

Regulated Power Supply(RPS) (0-30V),

Connecting wires etc.,

Circuit Diagram:

IN4723

+

_+

+

390Ω

A

_+

(0-50mA)

V A

RPS

(0-30V)

DMM

(0 -10V)

Fig. (a) V-I  Characteristics

(0-50mA)

+

_+

390Ω

A

+

V A

(0-10V)

RPS

(0-30V)

IN4723

RL

(0-1180 Ω)

+

_+

_+

+

_+

Fig. (b) Voltage Regulation Characteristics

Procedure:

1. Connect the circuit as per the circuit diagram Fig (a).

2. Vary the power supply so that  the voltage across the Zener diode  varies insteps of 0.5V(starting from zero) and  note down the corresponding current reading in the ammeter

3. Plot the graph Vz  Vs  Iz and Note the value of the Zener break down voltage from graph, which is the voltage at which the current of the diode increases rapidly.

4. To obtain the Zener resistance, calculate the inverse of slope of the steep position of the curve.

5. Connect the circuit as shown in Fig.(b).

6. Keep the RPS value at 20v,

7. Vary the Rheostat in steps and note down  VL and IL.

Observations:

| SL.. No. | V.I characteristics |  | Voltage regulation characteristics |  |
| --- | --- | --- | --- | --- |
|  | VZ(volts) | IZ(mA) | VL(volts) | IL(mA) |

Expected Graph:

VZK

IZ (mA)

VL

V-I Characteristics

VZ(Volts)

IL

Voltage Regulation Characteristics

Calculations:

Zener resistance = ∆VZ/∆IZ

%  Regulation = (VNL – VFL)/ VFL*100   , Where  VNL = No load voltage

VFL = Full load voltage

Result:

Viva-voce Questions:

- What are the applications of Zener diodes?

- What is Zener breakdown?

- How should Zener diode be connected with load for voltage regulation?

- What is the difference between Zener diode and p-n junction diode?

Rectifiers With Out Filters

Expt.No.2

Aim: To determine the Ripple factor and Load regulation characteristics of  Half Wave Rectifier (HWR), Full Wave Rectifier (FWR) and Bridge Rectifier.

Apparatus:

Rectifier circuit kits,

Step-down Transformers(0-12V,12V-0-12V)

Ammeters (0-100mA),

Digital Multi Meter(DMM),

Rheostat(0-1180)Ώ,

Dimmerstat(0-230v)

Connecting wires etc.,

Circuit Diagram:

Fig. (a) Half Wave Rectifier

230v

50 Hz

1- φ

+

A

_

(0-100mA)

DMM

(0-10V)

V A

_+

+

RL

(0-1180 Ω)

BY127

D

12V

+

A

_

(0-100mA)

230v

50 Hz

1- φ

V A

DMM

(0-10V)

+

_+

RL

(0-1180 Ω)

BY127

BY127

D2

Fig. (b) Full Wave Rectifier

D1

12V

12V

`

_

(0-100mA)

230v

50 Hz

1- φ

A

+

+

D1

D2

V A

DMM

(0-10V)

_

RL

(0-1180 Ω)

12V

D3

D4

Fig. (c) Bridge Rectifier

D1 to D4 are BY127 Diodes

Procedure:

- Connect the circuit as shown in Fig(a)

- First excluding load resistance from circuit, find no load voltage (VNL) which is a dc value.

- Keep the load resistance ( Rheostat ) at a maximum value i.e.  RL.

- By varying load resistance (RL) from maximum position, Idc can be varied 0 to 100mA in steps of 10mA and note down corresponding valus of Vdc and Vac .

- Draw the graph between Vdc on X-axis and   Idc on Y-axis.

- Repeat the same procedure for Fig.(b) and Fig.(c) also.

Calculations:

Ripple Factor ( r ) = Vac/Vdc

%  Regulation = (VNL – VFL)/ VFL*100   , Where  VNL = No load voltage

VFL = Full load voltage

Observations:

|  | Half Wave Rectifier (HWR) VNL = volts |  |  | Full Wave Rectifier (FWR), VNL = volts |  |  | Bridge Rectifier VNL = volts |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Idc) mA | Vac volts) | Vdc (volts) | Ripple Factor r =Vac/Vdc | Vac volts) | Vdc (volts) | Ripple Factor r =Vac/Vdc | Vac volts) | Vdc (volts) | Ripple Factor r =Vac/Vdc |

Expected Graph:

Vdc

Idc

Voltage Regulation Characteristics

FWR

BR

HWR

Result:

Viva-voce Questions:

- Which rectifier will give less ripple factor?

- Define PIV of a rectifier.

- What is the difference between efficiency and TUF?

BJT Characteristics  Of Common Emitter Configuration

Expt.No.3

Aim:  To determine the Input and Output characteristic of  BJT(CE Configuration) and to determine hybrid parameters of   B.J.T.

Apparatus:

BJT circuit kit,

Ammeters ( 0-50μA, 0-10mA),

Regulated Power Supply(RPS) (0-30V),

Connecting wires etc.,

Circuit Diagram:

RPS

(0-30v)

VBB

_+

+

V A

(0-1V)VBE

+

A

_+

(0-50μA) IB

A

++

(0-10mA)Ic

_+

+

V A

VCE(0-10V)

BC107

100kΩ

RPS

(0-30v)

VCC

E

B

C

_+

+

_+

Procedure:

For Input characteristic:

1. Connect the circuit as shown in Figure .

2. By varying VCC keep VCE =0 volts and now vary VBB in steps  and note down corresponding IB  and VBE.

3. Repeat step 2 for VCE=2 volts and tabulate the readings.

For output characteristic:

1. For the above connected circuit disconnect the input side voltmeter.

2. By varying VBB keep IB = 10A  and now vary VCC  in steps  and note down  corresponding IC and VCE.

4. Repeat the step 2 for  IB = 20 A and tabulate the readings.

Observations:

| Input characteristics |  |  |  | Output characteristics |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VCE=0V |  | VCE=2V |  | IB= 10 A |  | IB = 20 A |  |
| VBE(volts) | IB(A) | VBE(volts) | IB(A) | VCE(volts) | IC(mA) | VCE(volts) | IC(mA) |

Expected graph :

VBE,V

Ic

mA

VCE,V

Output characteristic

IB=20μA

IB=10μA

∆IC

∆VCE

VCE = 2v

IB,μA

mA

VCE = 0v

∆IB

∆VBE

Input characteristic

Ic

mA

VCE,V

50 μA

100 μA

200μA

Output characteristic

Input characteristic

Calculations:

Input Impedance, hie =  ∆VBE/∆IB ohms at  VCE   = Constant

Reverse Voltage Gain hre = ∆VBE/∆VCE  at IB = Constant

Forward Current Gain hfe = ∆IC/∆IB  at  VCE = Constant

Output Admittance hoe =  ∆IC/∆VCE  mhos at   IB = Constant

Result:

Precautions:

1. IB should not exceed 50 μA

2. VCE should not exceed 10v

3. Loose connections must be avoided.

Viva-voce Questions:

- What are the hybrid parameters of a BJT?

- Why BJT is called as bipolar device?

- Is BJT, a voltage controlled device or current controlled device?

- What are the different regions of operation in BJT?

- What are the applications of  BJT?

Field Effect Transistor(FET) Characteristics

Expt.No.4

Aim:  To obtain characteristics of the given JFET and to find the values of Drain resistance ‘rd’, Trans-conductance ‘gm’ and Amplification factor ‘μ’.

Apparatus:

JFET circuit kit,

Ammeters (0-10mA),

Voltmeter(0-10v),

Regulated Power Supply (RPS) (0-30V),

Connecting wires etc.,

Circuit Diagram:

0A79

V A

A

V A

RPS

(0-30v)

VGG

_

+

VGS

(0-10V)

++

(0-10mA)ID

_+

+

VDS

(0-10V)

BFW10

RPS

(0-30v)

VDD

S

G

D

_

_

+

+

_

Procedure:

1. Connect the circuit as shown in the figure .

2. Keep VGS = 0 volt and note down the current ID and drain to source voltage VDS by varying VDD.

3. Keep VGS = -1 by Varying VGG and repeat step 2.

4. Draw the drain characteristics by taking  VDS on X-axis and ID on Y-axis for different vaues of VGs.

5. keep VDS=2v by varying VDD and note down VGS and ID by varying VGG.

6. Repeat the same for VDS=4v  and plot the transfer characteristics by taking VGS on X-axis and ID on Y-axis.

Observations:

| Drain characteristics |  |  |  | Transfer characteristics |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VGS=0V |  | VGS= -1V |  | VDS=2V |  | VDS= 4V |  |  |
| VDS(volts) | ID(mA) | VDS(volts) | ID(mA) | VCE(volts) | IC(mA) |  | VCE(volts) | IC(mA) |

Expected graph :

ID( mA)

VDS(Volts)

Drain characteristic

VGS=0v

∆ID

∆VDS

VGS= -1v

VGS(V)

Transfer characteristic

ID

∆VGS

VDS=2v

∆ID

IDSS

IB,μA

mA

VBE,V

Ic

mA

VCE,V

50 μA

100 μA

200μA

Output characteristic

Input characteristic

Page 1 of 2

Calculations:

Drain resistance(rd) =  ∆ /∆ID ohms at  = Constant (10k to 25k ohms)

Trance conductance (gm)= ∆ID/∆VGS mhos at VDS = Constant(2m to 2.5 mhos)

Amplification Factor  (μ) = ∆IC/∆IB  at  VCE = Constant

Output Admittance hie =  gm* rd  (20 to 60)

Precautions:

1. ID should not exceed 8mA

2. VGS should not exceed -7v

Result:

Viva-voce Questions:

- Give the differences between FET and BJT?

- Define Drain resistance(rd), Trance conductance (gm), Amplification Factor(μ).

- Is BJT, a voltage controlled device or current controlled device?

- What are the different types of FET?

- What are the applications of the FET?

Expt.No.5

Aim: To plot the frequency of an inverting operational amplifier for a voltage gain of 10 and obtain the band width

Equipment: BNC daopters-2, CRO, RPS, Resident(1k & 10k), Oscillators, DMM

SLEWRATE: Slew rate is defined as the maximum rate of change of O/P voltage per unit of time and is usually specified in V/µsec

i.e SR=(dv/dt)/max v/ µsec

slew rate indicates how rapidly the o/p of an Op-Amp can change in response to changes in the I/P frequency with I/P amplitude constent

The slew rate changes with change in gain and is normally specified at unity gain and load. It decrease with increase in temperature

Circuit diagram:

10KΩ

Vcc

1KΩ

1KΩ

Vcc

~V1

741IC

2

3

4

7

V0

C

AV

(dB)

3dB

fI

fI- Comer frequency

Expected graph:

Procedure: